TELULAR CORPORATION

AT Command Reference

GSM SX5 Product Line

Distribution Date: 19 November 2002

# AT Commands according to V.25ter

| Command | Description |
| --- | --- |
| A/ | Re-issues last AT command given |
| ATA | Answer incoming call |
| ATD | Mobile Originated call to dialable number |
| ATD><MEM><N> | Originate call to phone number in memory <MEM> |
| ATD><N> | Originate call to phone number in current memory |
| ATD><STR> | Originate call to phone number in memory which corresponds to alphanumeric field <STR> |
| ATDL | Redial last telephone number used |
| ATE | Set command echo mode |
| ATH | Disconnect existing connection |
| ATI | Display product identification information |
| ATL | Set monitor speaker loudness |
| ATM | Set monitor speaker mode |
| ATO | Switch from command mode to data mode |
| ATP | Select pulse dialling |
| ATQ | Set Result code presentation mode |
| ATS0 | Set number of rings before automatically answering the call |
| ATS3 | Set command line termination character |
| ATS4 | Set response formatting character |
| ATS5 | Set command line editing character |
| ATS6 | Set pause before blind dialling |
| ATS7 | Set number of seconds to wait for connection completion |
| ATS8 | Set number of seconds to wait when comma dial modifier used |
| ATS10 | Set disconnect delay after indicating the absence of data carrier |
| ATT | Select tone dialling |
| ATV | Set result code format mode |
| ATX | Set connect result code format and call monitoring |
| ATZ | Set all current parameters to user defined profile |
| AT&C | Set DCD function mode |
| AT&D | Set DTR function mode |
| AT&F | Set all current parameters to manufacturer defaults |
| AT&V | Display current configuration |
| AT&W | Store current parameter to user defined profile |
| AT+DR | V.42bis data compression reporting control |
| AT+DS | V.42bis data compression control |
| AT+GCAP | Request complete TA capabilities list |
| AT+GMI | Request manufacturer identification |
| AT+GMM | Request TA model identification |
| AT+GMR | Request TA revision identification |
| AT+GOI | Request global object identification |
| AT+GSN | Request TA serial number identification (IMEI) |
| AT+ICF | Set TE-TA control character framing |
| AT+IFC | Set TE-TA local data flow control |
| AT+ILRR | Set TE-TA local rate reporting mode |
| AT+IPR | Set fixed local rate |

# AT Commands according to GSM 07.07

| Command | Description |
| --- | --- |
| AT+CACM | ACCUMULATED CALL METER(ACM) RESET OR QUERY |
| AT+CAMM | ACCUMULATED CALL METER MAXIMUM(ACMMAX) SET OR QUERY |
| AT+CAOC | ADVICE OF CHARGE |
| AT+CBST | SELECT BEARER SERVICE TYPE |
| AT+CCFC | CALL FORWARDING NUMBER AND CONDITIONS CONTROL |
| AT+CCUG | CLOSED USER GROUP CONTROL |
| AT+CCWA | CALL WAITING CONTROL |
| AT+CEER | EXTENDED ERROR REPORT |
| AT+CGMI | REQUEST MANUFACTURER IDENTIFICATION |
| AT+CGMM | REQUEST MODEL IDENTIFICATION |
| AT+CGMR | REQUEST REVISION IDENTIFICATION |
| AT+CGSN | REQUEST PRODUCT SERIAL NUMBER IDENTIFICATION (IDENTICAL WITH +GSN) |
| AT+CSCS | SELECT TE CHARACTER SET |
| AT+CSTA | SELECT TYPE OF ADDRESS |
| AT+CHLD | CALL HOLD AND MULTIPARTY |
| AT+CIMI | REQUEST INTERNATIONAL MOBILE SUBSCRIBER IDENTITY |
| AT+CKPD | KEYPAD CONTROL |
| AT+CLCC | LIST CURRENT CALLS OF ME |
| AT+CLCK | FACILITY LOCK |
| AT+CLIP | CALLING LINE IDENTIFICATION PRESENTATION |
| AT+CLIR | CALLING LINE IDENTIFICATION RESTRICTION |
| AT+CMEE | REPORT MOBILE EQUIPMENT ERROR |
| AT+COLP | CONNECTED LINE IDENTIFICATION PRESENTATION |
| AT+COPS | OPERATOR SELECTION |
| AT+CPAS | MOBIL EQUIPMENT ACTIVITY STATUS |
| AT+CPBF | FIND PHONEBOOK ENTRIES |
| AT+CPBR | READ CURRENT PHONEBOOK ENTRIES |
| AT+CPBS | SELECT PHONEBOOK MEMORY STORAGE |
| AT+CPBW | WRITE PHONEBOOK ENTRY |
| AT+CPIN | ENTER PIN |
| AT+CPWD | CHANGE PASSWORD |
| AT+CR | SERVICE REPORTING CONTROL |
| AT+CRC | SET CELLULAR RESULT CODES FOR INCOMING CALL INDICATION |
| AT+CREG | NETWORK REGISTRATION |
| AT+CRLP | SELECT RADIO LINK PROTOCOL PARAM. FOR ORIG. NON-TRANSP. DATA CALL |
| AT+CSQ | SIGNAL QUALITY REPORT |
| AT+FCLASS | FAX: SELECT, READ OR TEST SERVICE CLASS |
| AT+FMI | FAX: REPORT MANUFACTURED ID |
| AT+FMM | FAX: REPORT MODEL ID |
| AT+FMR | FAX: REPORT REVISION ID |
| AT+VTD | TONE DURATION |
| AT+VTS | DTMF AND TONE GENERATION |

# AT Commands according to GSM 07.05

| Command | Description |
| --- | --- |
| AT+CMGD | DELETE SMS MESSAGE |
| AT+CMGF | SELECT SMS MESSAGE FORMAT |
| AT+CMGL | LIST SMS MESSAGES FROM PREFERRED STORE |
| AT+CMGR | READ SMS MESSAGE |
| AT+CMGS | SEND SMS MESSAGE |
| AT+CMGW | WRITE SMS MESSAGE TO MEMORY |
| AT+CMSS | SEND SMS MESSAGE FROM STORAGE |
| AT+CMGC | SEND SMS COMMAND |
| AT+CNMI | NEW SMS MESSAGE INDICATIONS |
| AT+CPMS | PREFERRED SMS MESSAGE STORAGE |
| AT+CRES | RESTORE SMS SETTINGS |
| AT+CSAS | SAVE SMS SETTINGS |
| AT+CSCA | SMS SERVICE CENTER ADDRESS |
| AT+CSCB | SELECT CELL BROADCAST SMS MESSAGES |
| AT+CSDH | SHOW SMS TEXT MODE PARAMETERS |
| AT+CSMP | SET SMS TEXT MODE PARAMETERS |
| AT+CSMS | SELECT MESSAGE SERVICE |

# GPRS Commands

| Command | Description |
| --- | --- |
| +CGDCONT | Define PDP context |
| +CGQREQ | Quality of service profile (requested) |
| +CGQMIN | Quality of service profile (minimum acceptable) |
| +CGACT | Context activation |
| +CGDATA | Enter Data State |
| +CGATT | GPRS attach or detach |
| +CGPADDR | Show PDP address |
| +CGCLASS | GPRS mobile station class |
| +CGEREP | Control unsolicited GPRS event reporting |
| +CGREG | Network registration status |
| +CGSMS | Select service for MO SMS messages |
| +CGCOUNT | GPRS Packet Counters (TTPCom Proprietary) |

## GPRS Command Details

| Tab1 1 AT+CGDCONT Define the PDP context |  |
| --- | --- |
| Test command AT+CGDCONT=? | Response +CGDCONT: (range of supported <cid>s),<PDP_type>,,,(list of supported <d_comp>s),(list of supported <h_comp>s)[,(list of supported <pd1>s)[,…[, (list of supported <pdN>s)]]] [<CR><LF>+CGDCONT: (range of supported <cid>s),<PDP_type>,,,(list of supported <d_comp>s),(list of supported <h_comp>s)[,(list of supported <pd1>s)[,…[,(list of supported <pdN>s)]]] [...]] |
|  | Parameter See set command |
| Read command AT+CGDCONT? | Response +CGDCONT: <cid>,<PDP_type>,<APN>,<PDP_addr>,<data_comp>,<head_comp>[,<pd1>[,…[,pdN]]] [<CR><LF>+CGDCONT: <cid>,<PDP_type>,<APN>,<PDP_addr>,<data_comp>,<head_comp>[,<pd1>[,…[,pdN]]] [...]] |
|  | Parameter See set command |
| Set command AT+CGDCONT=[<cid>[,<PDP_type>[,<APN>[,<PDP_addr>[,<d_comp>[,<h_comp>[,<pd1>[,…[,pdN]]]]]]]]] | Response OK ERROR |
|  | Parameter <cid> (PDP Context Identifier) a numeric parameter which specifies a particular PDP context definition. The parameter is local to the TE-MT interface and is used in other PDP context-related commands. The range of permitted values (minimum value = 1) is returned by the test form of the command. <PDP_type> (Packet Data Protocol type) a string parameter which specifies the type of packet data protocol X25 ITU-T/CCITT X.25 layer 3 IP Internet Protocol (IETF STD 5) OSPIH Internet Hosted Octet Stream Protocol PPP Point to Point Protocol (IETF STD 51) <APN> (Access Point Name) a string parameter which is a logical name that is used to select the GGSN or the external packet data network. If the value is null or omitted, then the subscription value will be requested. <PDP_addr> a string parameter that identifies the MT in the address space applicable to the PDP. If the value is null or omitted, then a value may be provided by the TE during the PDP startup procedure or, failing that, a dynamic address will be requested. The read form of the command will continue to return the null string even if an address has been allocated during the PDP startup procedure. The allocated address may be read using the +CGPADDR command. <d_comp> a numeric parameter that controls PDP data compression 0 - off (default if value is omitted) 1 - on Other values are reserved. <h_comp> a numeric parameter that controls PDP header compression 0 - off (default if value is omitted) 1 – on Other values are reserved. NOTE. At present only one data compression algorithm (V.42bis) is provided in SNDCP. If and when other algorithms become available, a command will be provided to select one or more of these. <pd1>,…<pdN> zero to N string parameters whose meanings are specific to the <PDP_type> For PDP type OSP:IHOSS the following parameters are defined: <pd1> = <host> the fully formed domain name extended hostname of the Internet host <pd2> = <port > the TCP or UDP port on the Internet host <pd3> = <protocol> the protocol to be used over IP on the Internet - "TCP" or "UDP |
| Reference [4] | Note |

| Tab1 2 AT+CGQREQ Quality of service profile (requested) |  |
| --- | --- |
| Test command +CGQREQ=? | Response +CGQREQ: <PDP_type>,(list of supported <precedence>s),(list of supported <delay>s),(list of supported <reliability>s),(list of supported <peak>s),(list of supported <mean>s) [<CR><LF>+CGQREQ: <PDP_type>,(list of supported <precedence>s),(list of supported <delay>s),(list of supported <reliability>s),(list of supported <peak>s),(list of supported <mean>s) […]] |
|  | Parameter See set command |
| Read command +CGQREQ? | Response +CGQREQ: <cid>,<precedence>,<delay>,<reliability>,<peak>,<mean> [<CR><LF>+CGQREQ: <cid>,<precedence>,<delay>,<reliability>,<peak>,<mean> […]] |
|  | Parameter See set command |
| Set command +CGQREQ=[<cid>[,<precedence >[,<delay>[,<reliability.>[,<peak>[,<mean>]]]]]] | Response OK ERROR |
|  | Parameter <cid> a numeric parameter which specifies a particular PDP context definition (see +CGDCONT command). The following parameters are defined in GSM 03.60 - <precedence>a numeric parameter which specifies the precedence class <delay> a numeric parameter which specifies the delay class <reliability> a numeric parameter which specifies the reliability class <peak> a numeric parameter which specifies the peak throughput class <mean> a numeric parameter which specifies the mean throughput class |
| Reference [4] | Note |

| Tab1 3 AT+CGQMIN Quality of service profile (minimum acceptable) |  |
| --- | --- |
| Test command +CGQMIN=? | Response +CGQMIN: <PDP_type>,(list of supported <precedence>s),(list of supported <delay>s),(list of supported <reliability>s),(list of supported <peak>s),(list of supported <mean>s) [<CR><LF>+CGQMIN: <PDP_type>, (list of supported <precedence>s), (list of supported <delay>s),(list of supported <reliability>s),(list of supported <peak>s),(list of supported <mean>s) […]] |
|  | Parameter See set command |
| Read command +CGQMIN? | Response +CGQMIN: <cid>,<precedence>,<delay>,<reliability>,<peak>,<mean> [<CR><LF>+CGQMIN: <cid>,<precedence>,<delay>,<reliability>,<peak>,<mean> […]] |
|  | Parameter See set command |
| Set command +CGQMIN=[<cid>[,<precedence >[,<delay>[,<reliability>[,<peak>[,<mean>]]]]]] | Response OK ERROR |
|  | Parameter <cid> a numeric parameter which specifies a particular PDP context definition (see +CGDCONT command). The following parameters are defined in GSM 03.60 - <precedence>a numeric parameter which specifies the precedence class <delay> a numeric parameter which specifies the delay class <reliability> a numeric parameter which specifies the reliability class <peak> a numeric parameter which specifies the peak throughput class <mean> a numeric parameter which specifies the mean throughput class |
| Reference [4] | Note |

| Tab1 4 AT+CGACT PDP context activate or deactivate |  |
| --- | --- |
| Test command +CGACT=? | Response +CGACT: (list of supported <state>s) |
|  | Parameter See set command |
| Read command +CGACT? | Response +CGACT: <cid>,<state> [<CR><LF>+CGACT: <cid>,<state> […]] |
|  | Parameter See set command |
| Set command +CGACT=[<state>[,<cid>[,<cid>[,…]]]] | Response OK NO CARRIER ERROR |
|  | Parameter <state> indicates the state of PDP context activation 0 – deactivated 1 – activated Other values are reserved and will result in an ERROR response to the execution command. <cid> a numeric parameter which specifies a particular PDP context definition (see +CGDCONT command) |
| Reference [4] | Note If context is deactivated successfully, NO CARRIER is returned |

| Tab1 5 AT+CGDATA Enter Data State |  |
| --- | --- |
| Test command +CGDATA=? | Response +CGDATA: (list of supported <L2P>s) |
|  | Parameter See set command |
| Set command +CGDATA=[<L2P>[,<cid>[,<cid>[,…]]]] | Response OK ERROR |
|  | Parameter <L2P> a string parameter that indicates the layer 2 protocol to be used between the TE and MT: PPP – Point to Point protocol for a PDP such as IP Other values are not supported and will result in an ERROR response to the execution command. <cid> a numeric parameter which specifies a particular PDP context definition (see +CGDCONT command) |
| Reference [4] | Note The command does not fully implement the CGDATA command as specified in GSM 07.07. The command will not enter data state once the PDP context has been activated and will simply generate the result code “OK” if the context has been successfully activated. |

| Tab1 6 AT+CGATT GPRS attach or detach |  |
| --- | --- |
| Test command +CGATT=? | Response +CGATT: (list of supported <state>s) |
|  | Parameter See set command |
| Read command +CGATT? | Response +CGATT: <state> |
|  | Parameter See set command |
| Set command +CGATT= [<state>] | Response OK ERROR |
|  | Parameter <state> indicates the state of GPRS attachment 0 – detached 1 – attached Other values are reserved and will result in an ERROR response to the execution command. |
| Reference [4] | Note |

| Tab1 7 AT+CGPADDR Show PDP address |  |
| --- | --- |
| Test command +CGPADDR=? | Response +CGPADDR: (list of defined <cid>s) |
|  | Parameter See set command |
| Set command +CGPADDR=[<cid>[,<cid>[,…]]] | Response +CGPADDR: <cid>,<PDP_addr> [<CR><LF>+CGPADDR: <cid>,<PDP_addr> [...]] |
|  | Parameter <cid> a numeric parameter which specifies a particular PDP context definition (see +CGDCONT command). If no <cid> is specified, the addresses for all defined contexts are returned. <PDP_addr> a string that identifies the MT in the address space applicable to the PDP. The address may be static or dynamic. For a static address, it will be the one set by the +CGDCONT command when the context was defined. For a dynamic address it will be the one assigned during the last PDP context activation that used the context definition referred to by <cid>. <PDP_address> is omitted if none is available. |
| Reference [4] | Note This command dictates the behaviour of PPP in the ME but not that of any other GPRS-enabled foreground layer, eg browser. |

| Tab1 8 AT+CGCLASS GPRS mobile station class |  |
| --- | --- |
| Test command +CGCLASS=? | Response +CGCLASS: (list of supported <class>s) |
|  | Parameter See set command |
| Read command +CGCLASS? | Response +CGCLASS: <class> |
|  | Parameter See set command |
| Set command +CGCLASS= [<class>] | Response OK ERROR |
|  | Parameter <class> a string parameter which indicates the GPRS mobile class (in descending order of functionality) A class A (highest) B class B CG class C in GPRS only mode CC class C in circuit switched only mode (lowest) |
| Reference [4] | Note Class A and is not supported by the TTPCom GPRS solution. Class C is only supported for <class> values of “CG” and “CC” |

| Tab1 9 AT+CGEREP Control unsolicited GPRS event reporting |  |
| --- | --- |
| Test command +CGEREP=? | Response +CGEREP: (list of supported <mode>s),(list of supported <bfr>s) |
|  | Parameter See set command |
| Read command +CGEREP? | Response +CGEREP: <mode>,<bfr> |
|  | Parameter See set command |
| Set command +CGEREP=[<mode>[,<bfr>]] | Response OK ERROR |
|  | Parameter <mode> 0 buffer unsolicited result codes in the MT; if MT result code buffer is full, the oldest ones can be discarded. No codes are forwarded to the TE. 1 discard unsolicited result codes when MT-TE link is reserved (e.g. in on-line data mode); otherwise forward them directly to the TE 2 buffer unsolicited result codes in the MT when MT-TE link is reserved (e.g. in on-line data mode) and flush them to the TE when MT-TE link becomes available; otherwise forward them directly to the TE <bfr> 0 MT buffer of unsolicited result codes defined within this command is cleared when <mode> 1 or 2 is entered 1 MT buffer of unsolicited result codes defined within this command is flushed to the TE when <mode> 1 or 2 is entered (OK response shall be given before flushing the codes) Unsolicited Result Codes supported: +CGEV: NW DEACT <PDP_type>,<PDP_addr>[,<cid>] +CGEV: ME DEACT <PDP_type>,<PDP_addr>[,<cid>] +CGEV: NW DETACH +CGEV: ME DETACH +CGEV: ME CLASS <class> Parameter <PDP_type> Packet Data Protocol type (see +CGDCONT command) <PDP_addr> Packet Data Protocol address (see +CGDCONT command} <cid> Context Id (see +CGDCONT command) Note: <cid> only given if known to the MT. <class> GPRS mobile class (see +CGCLASS command) |
| Reference [4] | Note |

| Tab1 10 AT+CGREG Network registration status |  |
| --- | --- |
| Test command +CGREG=? | Response +CGREG: (list of supported <n>s) |
|  | Parameter See set command |
| Read command CGREG? | Response +CGREG: <n>,<stat>[,<lac>,<ci>] +CME ERROR: <err> |
|  | Parameter See set command |
| Set command +CGREG=[<n>] | Response |
|  | Parameter <n> 0 disable network registration unsolicited result code - enable network registration unsolicited result code +CGREG: <stat> 2 enable network registration and location information unsolicited result code +CGREG: <stat>[,<lac>,<ci>] <stat> - not registered, ME is not currently searching a new operator to register to - registered <lac> string type; two byte location area code in hexadecimal format (e.g. "00C3" equals 195 in decimal) <ci> string type; two byte cell ID in hexadecimal format |
| Reference [4] | Note For parameter stat, options 0 and 1 supported only. |

| Tab1 11 AT+CGSMS Select service for MO SMS messages |  |
| --- | --- |
| Test command +CGSMS=? | Response +CGSMS: (list of currently available <service>s) |
|  | Parameter See set command |
| Read command +CGSMS? | Response +CGSMS: <service> |
|  | Parameter See set command |
| Set command +CGSMS= [<service>] | Response OK ERROR |
|  | Parameter <service> a numeric parameter which indicates the service or service preference to be used - GPRS 1 circuit switched 2 GPRS preferred (use circuit switched if GPRS not available) 3 circuit switched preferred (use GPRS if circuit switched not available) |
| Reference [4] | Note The circuit switched service route is the default method |

| Tab1 12 AT+CGCOUNT |  |
| --- | --- |
| Test command +CGCOUNT=? | Response +CGCOUNT: (list of supported <actions>s),(list of supported <cid>s),(list of supported <period>s) |
|  | Parameter See set command |
| Read command +CGCOUNT? | Response +CGCOUNT: <cid>,<state>[,<period>] [<CR><LF>+CGCOUNT: <cid>,<state>[,<period>] […]] |
|  | Parameter <state> indicates the state of the GPRS counters 1 – periodic. The <period> will then also be displayed 2 – on GPRS context deactivation. <period> is N/A in this case For other parameters see set command |
| Set command +CGCOUNT=<action>,<cid>[,<period>] | Response OK ERROR |
|  | Parameter <action> indicates the action to be performed 0 – reset counter for specified <cid> - – read counter for specified <cid> - – start reporting counter periodically for specified <cid> defined by <period>. Counter is also reported on context deactivation. - – report counter on context deactivation for specified <cid> - – stop reporting counter on specified <cid> <cid> a numeric parameter which specifies a particular PDP context definition (see +CGDCONT command) <period> period for periodic packet counter reporting in seconds Unsolicited Result Once a counter has been setup for a <cid> the counter will be displayed as Following either periodically or when the context has been deactivated: +CGCOUNT: <cid>,<uc>,<uu>,<un>,<dc>,<du>,<dn> <uc> a numeric 32 bit parameter which indicates the number of compressed bytes transferred in the uplink direction displayed in decimal format <uu> a numeric 32 bit parameter which indicates the number of uncompressed bytes transferred in the uplink direction displayed in decimal format <un> a numeric 32 bit parameter which indicates the number of N-PDUs (i.e. IP packets) transferred in the uplink direction displayed in decimal format <dc> a numeric 32 bit parameter which indicates the number of compressed bytes transferred in the downlink direction displayed in decimal format <du> a numeric 32 bit parameter which indicates the number of uncompressed bytes transferred in the downlink direction displayed in decimal format <dn> a numeric 32 bit parameter which indicates the number of N-PDUs (i.e. IP packets) transferred in the downlink direction displayed in decimal format Note that the current counter values will be displayed immediately this command is entered for any action (i.e. even stopping the counter display will generate the above unsolicited result code for the cancelled <cid>). |
| Reference [4] | Note This command displays byte and IP packet counters for GPRS contexts. It is proprietary to TTPCom. If counters are displayed periodically, they will only be displayed if: - there is a separate multiplexer channel for unsolicited result codes, or - the user switches to command mode using the “+++” escape sequence |

# SIM Application Tool Kit Commands

| Notation | Description |
| --- | --- |
| +STC: | Unsolicited result code issued by the CI Task to the application to indicate either: - there is no STK application available on the SIM - there is a proactive SIM command to retrieve and action - end of the current proactive command session – used if the user wishes to terminate the current proactive SIM session. |
| +STGC=… | AT command to Get Command parameters for a proactive SIM command from the CI Task. This will be sent from the application after unsolicited result code +STC: <cmdId> informs it the SIM has issued a proactive SIM command to be performed. |
| +STCR=… | AT command to provide Command Response parameters for a previously executed proactive SIM command. Its purpose is to relay response data to the lower layers of the TTPCom protocol stack to allow the Terminal Response SIM command (see [10]) to be returned to the SIM for the current proactive command. |
| +STPD=… | AT command to provide Profile Download parameters to the CI Task. This contains information relating to the SIM Application Toolkit capabilities of the application, and is used by the SIMAT task to limit its SAT instruction set accordingly. Any application plugging into the serial port should send this command or it will be assumed that the application has no SAT support and will therefore never receive any SAT related information. |
| +STMS=… | AT Command for selecting a menu option. On power-up the SIM will send the Set-Up-Menu proactive indication. The accessory should load and display the menu structure. This AT command should be used to inform Voyager of the item selected from the list. |
| +STEV=… | This command is used to inform the MS that an MMI specific event has occurred. |
| +STRT=… | AT command for setting the automatic response timer used by the CI Task to issue the Terminal Response (no user response) to a proactive command which has not been processed. The default response time is ten seconds, but it is recommended this is increased when performing SIM Toolkit FTA. |
| +STTONE=… | AT command for playing SIM Toolkit Tones in both idle and dedicated mode. This command should be used in conjunction with the Play Tone proactive command. |

## Definition of Unsolicited Result Codes

Not all proactive commands are required to be visible to the application.  For example, the proactive commands More Time and Provide Local Information are transparent and therefore do not require an unsolicited result code to be sent to the user. The commands, which are relevant for user interaction in one form or another, are listed in the following tables.

The output generated for strings is controlled by the +CMGF AT command.  The factory default for string output is PDU mode where strings are output in HEX.  The tables below illustrate the alternative mechanism of TEXT output; this is obtained by using the +CMGF AT command with a parameter of one.

### +STC Command

| Tab3 1 +STC Informs the application of the type of proactive SIM command data awaiting retrieval. |  |
| --- | --- |
| Execute command +STC: <cmdId> | Parameters <cmdId> Hexadecimal format of Type of Command . Unique identifier for the current SIM Toolkit proactive command issued by the SIM - see [9]. The following values are supported: ‘10’ Get Acknowledgement For Set Up Call command ‘15’ Launch Browser command ‘20’ Play Tone command ‘21’ Display Text command ‘22’ Get Inkey command ‘23’ Get Input command ‘24’ Select Item command ‘25’ Set Up Menu command ‘28’ Set Up Idle Mode Text command ‘40’ Open Channel command ‘14’ Send DTMF command ‘05’ Set Up Event List command ‘81’ End of proactive session |
| Reference | Note The special case is +STC: 0 that is issued when there is no STK application accessible on the SIM. |

The following tables in this section detail the information that is distributed to the application for proactive indications using unsolicited result codes.  The information applicable to the proactive command is sent to the application using the +STUD (SIM Toolkit Unsolicited Data) results code.

### Send SM

| Tab3 2 Command data for Send Short Message unsolicited proactive command |  |
| --- | --- |
| Result Code +STUD: 13[,<alphaId>[,<iconId>,<dispMode>]] | Parameters 13 hex notation: Command Type value. See Section 5.2 for values. <alphaId> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB). ‘0’ : Special case indicating SIM provided a null alphaId and user should not be informed of SMS transaction. If alphaId field is not present it is up to the ME to decide whether to inform the user or not. <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon 0 display icon only (replaces any text string or alphaId) - display with alphaId or text string |
| Reference | Note |

### Send SS

| Tab3 3 Command data for Send SS unsolicited proactive command |  |
| --- | --- |
| Result Code +STUD: 11[,<alphaId>[,<iconId>,<dispMode>]] | Description 11 hex notation: Command Type value. See Section 5.2 for values. <alphaId> string format: using either SMS default alphabet (see [10]) or UCS2 alpha field coding (see [12] AnnexB) to inform user of current transaction. ‘0’ : Special case indicating SIM provided a null alphaId and user should not be informed of SS transaction. If alphaId field is not present it is up to the ME to decide whether to inform the user or not. <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon 0 display icon only (replaces any text string or alphaId) 1 display with alphaId or text string |
| Reference | Note |

### Send USSD

| Tab3 4 Command data for Send USSD unsolicited proactive command |  |
| --- | --- |
| Result Code +STUD: 12[,<alphaId>[,<iconId>,<dispMode>]] | Parameters 12 hex notation: Command Type value. See Section 5.2 for values. <alphaId> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) to inform user of current transaction. ‘0’ : Special case indicating SIM provided a null alphaId and user should not be informed of USSD transaction. If alphaId field is not present it is up to the ME to decide whether to inform the user or not. <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon 0 display icon only (replaces any text string or alphaId) 1 display with alphaId or text string |
| Reference | Note |

### Set Up Call

| Tab3 5 Command data for Set Up Call unsolicited proactive command |  |
| --- | --- |
| Result Code +STUD: 10,<alphaId>,<dialstring>,<cps>[,<iconId>,<dispMode>] | Parameters 10 hex notation: Command Type value. See Section 5.2 for values. <alphaId> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) <dialstring> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) <cps> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon 0 display icon only (replaces any text string or alphaId) 1 display with alphaId or text string |
| Reference | Note |

### Close Channel

| Tab3 7 Command data for Close Channel proactive command |  |
| --- | --- |
| Result Code +STUD: 41[,<alphaId>[,<iconId>,<dispMode>]] | Parameters 41 hex notation: Command Type value. See Section 5.2 for values. <alphaId> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) to inform user of current transaction. ‘0’ : Special case indicating SIM provided a null alphaId and the user should not be informed of the current transaction. If alphaId field is not present it is up to the ME to decide whether or not to inform the user. <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon 0 display icon only (replaces any text string or alphaId) 1 display with alphaId or text string |
| Reference | Note |

### Receive Data

| Tab3 8 Command data for Receive Data proactive command |  |
| --- | --- |
| Result Code +STUD: 42,<length>[,<alphaId>[,<iconId>,<dispMode>]] | Parameters 42 hex notation: Command Type value. See Section 5.2 for values. <length> integer type: number of bytes requested in command <alphaId> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) to inform user of current transaction. ‘0’ : Special case indicating SIM provided a null alphaId and the user should not be informed of the current transaction. If alphaId field is not present it is up to the ME to decide whether or not to inform the user. <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon 0 display icon only (replaces any text string or alphaId) 1 display with alphaId or text string |
| Reference | Note |

### Send Data

| Tab3 9 Command data for Send Data proactive command |  |
| --- | --- |
| Result Code +STUD: 43,<length>,<data>[,<alphaId>[,<iconId>,<dispMode>]] | Parameters 43 hex notation: Command Type value. See Section 5.2 for values. <length> integer type: number of bytes of data transmitted <data> string type: channel data – coded as 8bit data. This appears in BCD notation with two TE characters representing one byte of actual data <alphaId> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) to inform user of current transaction. ‘0’ : Special case indicating SIM provided a null alphaId and the user should not be informed of the current transaction. If alphaId field is not present it is up to the ME to decide whether or not to inform the user. <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon 0 display icon only (replaces any text string or alphaId) 1 display with alphaId or text string |
| Reference | Note |

### Language Notification

| Tab3 10 Command data for Language Notification proactive command |  |
| --- | --- |
| Result Code +STUD: 35[,<language>] | Parameters 35 hex notation: Command Type value. See Section 5.2 for values. <language> language code: coded as pair of alphanumeric characters, as given in ISO 639 [12]. |
| Reference | Note The language parameter is optional. Its inclusion in the result code indicates a specific language notification. Omission from the result code indicates a non-specific language notification, which cancels a previous specific language notification |

### Run AT

| Tab3 11 Command data for Run AT Command proactive command |  |
| --- | --- |
| Result Code +STUD: 34[,<alphaId>[,<iconId>,<dispMode>]] | Parameters 34 hex notation: Command Type value. See Section 5.2 for values. <alphaId> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) to inform user of current transaction. ‘0’ : Special case indicating SIM provided a null alphaId and the user should not be informed of the current transaction. If alphaId field is not present it is up to the ME to decide whether or not to inform the user. <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon 0 display icon only (replaces any text string or alphaId) 1 display with alphaId or text string |

### Refresh

| Tab3 13 Command data for Refresh proactive command |  |
| --- | --- |
| Result Code +STUD: 01,<refMode>[,<numFiles>,<fileList>] | Parameters 01 hex notation: Command Type value. See Section 5.2 for values. <refMode > hex notation: command Qualifier information giving the type of Refresh to be performed. - SIM Initialisation and Full File Change Notification - File Change Notification - SIM Initialisation and File Change Notification - SIM Initialisation - SIM Reset <numFiles> integer: gives number of Files in the list <fileList> string type, hex notation: gives the full paths for the SIM files, each file being delimited by commas within the string |
| Reference | Note For <refMode> values ’01’ and ‘02’ file list data must be provided by the SIM. For all other <refMode> values any included file list information will be ignored. If the optional <fileList> parameter is not present in the result code, we assume that <refMode>s ’01’and ‘02’ cannot occur. |

#### AT commands for driving an STK application on the SIM.

### AT+STGC SIM Toolkit Get Command parameters

| Tab4 1 Get proactive Command parameters |  |
| --- | --- |
| Set command +STGC=<cmdId> | Response +STGC: <cmdId>,<data> |
|  | Parameter <cmdId> hex notation: Command Type value See Section 5.2 for values. <data> proactive command specific data, dependent on <cmdId> |
| Reference | Note |

The <data> information varies between proactive SIM commands, according to the type of command issued by the SIM, as given by <cmdId>. This reflects the useful part of the proactive command from a user’s perspective. The result codes returned to the application on a command by command basis are outlined in the following subsections:

#### Display Text

| Tab4 2 Command data for Display Text proactive command |  |
| --- | --- |
| Result Code +STGC: 21,<dcs>,<text>,<priority>,<clear>[,<iconId>,<dispMode>[,<response>]] | Parameters 21 hex notation: Command Type value. See Section 5.2 for values. <dcs > integer: data coding scheme used for <text>. The schemes used are as per GSM 03.38 for SMS (see [11]). - 7bit GSM default alphabet (packed) 4 8bit data 8 UCS2 alphabet <text> string format: text string in <dcs> format <priority> integer: display priority information 0 Normal priority 1 High priority <clear> integer: mode of clearing message 0 Clear after delay 1 User clears message <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon - Display icon only (replaces any text string or alphaId) 1 display with alpha Id or text string <response> 0 normal reponse expected 1 immediate response expected. |
| Reference | Note |

#### Get InKey

| Tab4 3 Command data for Get Inkey proactive command |  |
| --- | --- |
| Result Code +STGC: 22,<dcs>,<text>,<response>,<helpInfo>[,<iconId>,<dispMode>] | Parameters 22 hex notation: Command Type value. See Section 5.2 for values. <dcs > integer: data coding scheme used for <text> The schemes used are as per GSM 03.38 for SMS (see [11]). 0 7bit GSM default alphabet (packed) 4 8bit data 8 UCS2 alphabet <text> string format: text string in <dcs> format <response> integer: expected response character format. - Digits (0-9, *, # and +) only - SMS default alphabet - UCS2 alphabet - Yes/No response only <helpInfo> 0 no help information available 1 help information available <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) - No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon - display icon only (replaces any text string or alphaId) 1 display with alpha Id or text string |
| Reference | Note Entry of the Digits only response is the same regardless of alphabet set – coding of this response is performed within the TTPCom Protocol Stack when creating the Terminal Response |

#### Get Input

| Tab4 4 Command data for Get Input proactive command |  |
| --- | --- |
| Result Code +STGC: 23,<dcs>,<text>,<response>,<echo>,<helpInfo>,<minLgth>,<maxLgth>[,<dcs>,<default>[,<iconId>,<dispMode>]] | Parameters 23 hex notation: Cmmand Type value. See Section 5.2 for values. <dcs > integer: data coding scheme used for <text> or <default>. The schemes used are as per GSM 03.38 for SMS (see [11]). 0 7bit GSM default alphabet (packed) 4 8bit data 8 UCS2 alphabet <text> string format: text string in <dcs> format <response> integer: expected response characters and their format. - Digits (0-9, *, # and +) only from SMS default alphabet (unpacked) - Digits (0-9, *, # and +) only from SMS default alphabet (packed) - Digits from UCS2 alphabet - SMS default alphabet (unpacked) - SMS default alphabet (packed) - UCS2 alphabet <echo> 0 echo input to display 1 no echo allowed (see Note) <helpInfo> 0 no help information available 1 help information available <minLgth> Integer: minimum length of expected response, in range 0..255 0 indicates no minimum length requirement <maxLgth> Integer: maximum length of expected response, in range 1..255 255 indicates no maximum length requirement <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon - display icon only (replaces any text string or alphaId) 1 display with alpha Id or text string |
| Reference | Note Actual input string may not be displayed in this case but can alternatively be masked to indicate key entry using characters from the set (0-9, * and #). If <minLgth> and <maxLgth> are equal, the response string is to be of fixed length. |

#### PlayTone

| Tab4 5 Command data for Play Tone proactive command |  |
| --- | --- |
| Result Code +STGC: 20[,<alphaId>[,<tone>[,<duration>]]] | Parameters 20 hex notation: Command Type value. See Section 5.2 for values. <alphaId> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) <tone> integer: identifies requested tone type. SST denotes a Standard Supervisory Tone, MPT denotes an ME Proprietary Tone. - Dial (SST) - Called subscriber busy (SST) - Congestion (SST) - Radio Path acknowledge (SST) - Radio path not available / Call dropped (SST) - Error / Special information (SST) - Call waiting (SST) - Ringing Tone (SST) - General Beep (MPT) - Positive ack (MPT) - Negative ack or Error (MPT) <duration> integer: duration of the tone to be played, given in milliseconds. |
| Reference | Note If no tone is specified the ME shall default to the General Beep SST. If no duration is specified the ME default of 500ms is chosen. |

#### Set Up Menu

| Tab4 6 Command data for Set Up Menu proactive command |  |
| --- | --- |
| Result Code +STGC: 25,<numItems>,<selection>,<helpInfo>,<alphaId>[,<iconId>,<dispMode>]<CR><LF> +STGC: <itemId>,<itemText>[,<iconId>,<dispMode>,<nai><CR><LF> [+STGC: <itemId>,<itemText>[,<iconId>,<dispMode>,<nai><CR><LF> […]]]] | Parameters 25 hex notation: Command Type value. See Section 5.2 for values. <numItems> integer: indicates the number of items accessible in the menu structure. 0 is a special case, indicating the existing menu is to be removed from the ME’s menu structure. <selection> integer: gives preferred user selection method 0 no selection preferrence 1 soft key selection preferred <helpInfo> 0 no help information available - help information available <alphaId> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon 0 display icon only (replaces any text string or alphaId) - display with alpha Id or text string <itemId> integer: denotes the identifier of the item <itemText> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) <nai> hex notation: next action indicator – this takes one of the allowed values from the Command Type (see section 5.2) range, as specified in [9], section 13.4 |
| Reference | Note |

#### Select Item

| Tab4 7 Command data for Select Item proactive command |  |
| --- | --- |
| Result Code +STGC: 24,<numItems>,<selection>,<helpInfo>,<alphaId>[,<iconId>,<dispMode>]<CR><LF> +STGC: <itemId>,<itemText>[,<iconId>,<dispMode>,<nai><CR><LF> [+STGC: <itemId>,<itemText>[,<iconId>,<dispMode>,<nai><CR><LF> […]]]] | Parameters 24 hex notation: Command Type value. See Section 5.2 for values. <numItems> integer: indicates the number of items accessible in the menu structure. 0 is a special case, indicating the existing menu is to be removed from the ME’s menu structure. <selection> integer: gives preferred user selection method 0 no selection preferrence 1 soft key selection preferred <helpInfo> 0 no help information available 1 help information available <alphaId> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon 0 display icon only (replaces any text string or alphaId) - display with alpha Id or text string <itemId> integer: denotes the identifier of the item <itemText> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) <nai> hex notation: next action indicator – this takes one of the allowed values from the Command Type (see section 5.2) range, as specified in [9], section 13.4 |
| Reference | Note |

#### Get Acknowledgement For Set Up Call

| Tab4 8 Command data for Set Up Call proactive command |  |
| --- | --- |
| Result Code +STGC: 10,<alphaId>[,<iconId>,<dispMode>] | Parameters 10 hex notation: Command Type value. See Section 5.2 for values. <alphaId> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon 0 display icon only (replaces any text string or alphaId) 1 display with alphaId or text string |
| Reference | Note |

#### Set Up Idle Mode Text

| Tab4 9 Command data for Set Up Idle Mode Text proactive command |  |
| --- | --- |
| Result Code +STGC: 28,<dcs>,<text>[,<iconId>,<dispMode>] | Parameters 28 hex notation: Command Type value. See Section 5.2 for values. <dcs > integer: data coding scheme used for <text>. The schemes used are as per GSM 03.38 for SMS (see [11]). 0 7bit GSM default alphabet (packed) 4 8bit data 8 UCS2 alphabet <text> string format: text string in <dcs> format See Note below. <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon 0 display icon only (replaces any text string or alphaId) 1 display with alphaId or text string |
| Reference | Note If the text string given in the result code is Null (i.e. zero length and set as “” in the result code) it implies the existing Idle Mode Text is to be removed. |

#### Send DTMF

| Tab4 10 Command data for Send DTMF proactive command |  |
| --- | --- |
| Result Code +STGC: 14[,<alphaId>[,<iconId>,<dispMode>]] | Parameters 14 hex notation: Command Type value. See Section 5.2 for values. <alphaId> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) to inform user of current transaction. ‘0’ : Special case indicating SIM provided a null alphaId and the user should not be informed of the current transaction. If alphaId field is not present it is up to the ME to decide whether or not to inform the user. <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon 0 display icon only (replaces any text string or alphaId) 1 display with alphaId or text string |
| Reference | Note |

#### Launch Browser

| Tab4 11 Command data for Launch Browser proactive command |  |
| --- | --- |
| Result Code +STGC: 15,<comQual>,<url>[,<browserId>[,<bearer>[,<numFiles>,<provFiles>[,<dcs>,<gateway>[,<alphaId>[,<iconId>,<dispMode>]]]]]] | Parameters 15 hex notation: Command Type value. See Section 5.2 for values. <comQual> hex notation: command qualifier information from Command Details Data Object: - launch browser without making connection, if not already launched - launch browser making connection, if not already launched - use existing browser - close existing browser, launch new browser, making a connection - close existing browser, launch new browser, using secure session <url> string format: 8bit data using GSM default 7bit alphabet. Special case: <url>=”” – Null value, so use default URL <browserId> hex notation: Browser Id to use. Available values: ‘00’ Use default browser <bearer> hex notation: list of allowed bearers in priority order. Possible values: ‘00’ SMS ‘01’ CSD ‘02’ USSD ‘03’ GPRS <numFiles> integer: denotes the number of provisioning files given <provFiles> string type, hex notation file ids: List of Provisioning File Reference ids. Full Paths are given, delimeted within the string by a comma <dcs > integer: data coding scheme used for <text>. The schemes used are as per GSM 03.38 for SMS (see [11]). 0 7bit GSM default alphabet (packed) 4 8bit data 8 UCS2 alphabet <gateway> string format: text string in <dcs> format <alphaId> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon 0 display icon only (replaces any text string or alphaId) 1 display with alphaId or text string |
| Reference | Note |

#### Open Channel

| Tab4 12 Command data for Open Channel proactive command |  |
| --- | --- |
| Result Code +STGC: 40[,<alphaId>[,<iconId>,<dispMode>]] | Parameters 40 hex notation: Command Type value. See Section 5.2 for values. <alphaId> string format: using either SMS default alphabet (see [11]) or UCS2 alpha field coding (see [10] AnnexB) to inform user of current transaction. ‘0’ : Special case indicating SIM provided a null alphaId and the user should not be informed of the current transaction. If alphaId field is not present it is up to the ME to decide whether or not to inform the user. <iconId> Numeric tag for the icon to be displayed – corresponds to the index in the Image file on the SIM (see [10]) 0 No icon 1..255 Icon tag <dispMode> integer: denotes use of associated icon 0 display icon only (replaces any text string or alphaId) 1 display with alphaId or text string |
| Reference | Note |

#### Set Up Event List

| Tab4 13 Command data for Set Up Event List proactive command |  |
| --- | --- |
| Result Code +STGC: 05,<eventList> | Parameters 05 hex notation: Command Type value. See Section 5.2 for values. <eventList> hex: denotes applicable event identifiers. 05 User activity event 06 Idle Screen Available event 08 Language Selection event 09 Browser termination event FF Remove existing event list |
| Reference | Note <eventList> value of FF used to remove existing list of events as value 0 can be confused with event MT Call value. This command causes the application to send a GSM 11.14 [9] ENVELOPE (EVENT DOWNLOAD) command to the SIM. |

### AT+STCR SIM Toolkit Command Response

Once a proactive command has been processed by the application a response needs to be sent to the SIM in the form of a TERMINAL RESPONSE command (see [9] and [10] for further information). It is therefore only a requirement for the application to issue command +STCR for those proactive commands it already retrieved via the +STGC AT command. The general format is shown below:

| Tab5 1 AT+STCR SIM Toolkit Command Response data |  |
| --- | --- |
| Set command +STCR=<cmdId>,<result>[,<data>] | Response +CME ERROR: <err> |
|  | Parameter <result> hex notation: dependent on the command type – see following sections for each proactive command supported. The values given in the result field for each set of proactive command response parameters the setting of the general result parameter returned to the SIMAT task in the next phase of signaling for building the Terminal Response command. <data> additional data provided for certain commands, as required for the Terminal Response returned to the SIM after processing a proactive SIM command |
| Reference [10] | Note |

For the above AT Command, the data contained within the <data> field varies depending on the current proactive SIM command being processed. The result data available for each of the proactive commands processed by the application is described in the following subsections:

#### Display Text

| Tab5 2 Command response for Display Text proactive command |  |
| --- | --- |
| Execute command +STCR=21,<result> | Parameters 21 hex notation: Command Type value. See Section 5.2 for values. <result> integer: possible values: - Message displayed OK - Terminate proactive session - User cleared message - Screen is busy - Backward move requested - No response from user |
| Reference | Note |

#### Get InKey

| Tab4 3 Command response for Get Inkey proactive command |  |
| --- | --- |
| Execute command +STCR=22,<result>[,<dcs>,<text>] | Parameters 22 hex notation: Command Type value. See Section 5.2 for values. <result> integer: possible values: - Data entered OK - Terminate proactive session - Help information requested - Backward move requested - No response from user <dcs > integer: data coding scheme used for <text>. The schemes used are as per GSM 03.38 for SMS (see [11]). 0 7bit GSM default alphabet (packed) 4 8bit data 8 UCS2 alphabet <text> string format: text string in <dcs> format Special cases are: “00” Negative response entered “01” Positive response entered |
| Reference | Note The <dcs> and <text> information must be provided for <result>=0 as the SIM expects the input to be provided in a Text String Data Object in the Terminal Response SIM command when data has been input. |

#### Get Input

| Tab5 4 Command response for Get Input proactive command |  |
| --- | --- |
| Execute command +STCR=23,<result>[,<dcs>,<text>] | Parameters 23 hex notation: Command Type value. See Section 5.2 for values. <result> integer: possible values: - Data entered OK - Terminate proactive session - Help information requested - Backward move requested - No response from user <dcs > integer: data coding scheme used for <text>. The schemes used are as per GSM 03.38 for SMS (see [11]). 0 7bit GSM default alphabet (packed) 4 8bit data 8 UCS2 alphabet <text> string format: text string in <dcs> format |
| Reference | Note If the <dcs> is present but <text> is an empty string this indicates a null text string data object must be sent to the SIM. This is caused by the user making an ‘empty’ input. |

#### PlayTone

| Tab5 5 Command response for Play Tone proactive command |  |
| --- | --- |
| Execute command +STCR=20,<result> | Parameters - hex notation: Command Type value. See section 5.2 for values. <result> integer: possible values: - Command performed OK - Terminate proactive session - Tone not played - Specified tone not supported |
| Reference | Note |

#### Set Up Menu

| Tab5 6 Command response for Set Up Menu proactive command |  |
| --- | --- |
| Execute command +STCR=25,<result> | Parameters 25 hex notation: Command Type value. See Section 5.2 for values. <result> integer: possible values: - Menu successfully added/removed - User chosen menu item - Help information requested - Problem with menu operation |
| Reference | Note |

#### Select Item

| Tab5 7 Command response for Select Item proactive command |  |
| --- | --- |
| Execute command +STCR=24,<result>[,<itemId>] | Parameters 24 hex notation: Command Type value. See Section 5.2 for values. <result> integer: possible values: - Item Selected OK - Terminate proactive session - Help information requested - Backward move requested - No response given <itemId> integer: denotes identifier of item selected |
| Reference | Note |

#### Get Acknowledgement For Set Up Call

| Tab5 8 Command response for Set Up Call proactive command |  |
| --- | --- |
| Execute command +STCR=10,<result> | Parameters 10 hex notation: Command Type value. See Section 5.2 for values. <result> integer: possible values: - user accepted call (conf phase only) - user rejected call (conf phase only) - user cleared call (any phase) |
| Reference | Note |

#### Set Up Idle Mode Text

| Tab5 10 Command response for Set Up Idle Mode Text proactive command |  |
| --- | --- |
| Execute command +STCR=28,<result> | Parameters 28 hex notation: Command Type value. See Section 5.2 for values. <result> integer: possible values: - Text successfully added/removed - Problem performing command |
| Reference | Note |

#### Launch Browser

| Tab5 11 Command response for Launch Browser proactive command |  |
| --- | --- |
| Execute command +STCR=15,<result> | Parameters 15 hex notation: Command Type value. See Section 5.2 for values. <result> integer: possible values: - Command performed successfully - Command performed – partial comp - Command performed – missing info - User rejected launch - Error – no specific cause given - Bearer unavailable - Browser unavailable - ME cannot process command - Network cannot process command - Command beyond MEs capabilities |
| Reference | Note |

#### Open Channel

| Tab5 12 Command response for Open Channel proactive command |  |
| --- | --- |
| Execute command +STCR=40,<result> | Parameters 40 hex notation: Command Type value. See Section 5.2 for values. <result> integer: possible values: - Channel not accepted - Channel required |
| Reference | Note |

#### Send DTMF

| Tab5 13 Command response for Send DTMF proactive command |  |
| --- | --- |
| Execute command +STCR=13,<result> | Parameters 13 hex notation: Command Type value. See Section 5.2 for values. <result> integer: possible values: - DTMF not accepted - DTMF required |
| Reference | Note |

#### Set Up Event List

| Tab4 9 Command response for Set Up Event List proactive command |  |
| --- | --- |
| Execute command +STCR=05,<result> | Parameters 05 hex notation: Command Type value. See Section 5.2 for values. <result> integer: possible values: - Command performed successfully - Cannot perform command |
| Reference | Note |

### AT+STPD SIM Toolkit Profile Download

When an application is plugged into the serial port the SIMAT task needs to have knowledge of its SAT capabilities to enable it to route all SAT related signalling to that application if required. If this command is not received it will be assumed that any attached application has no SAT capability and will therefore not send any related signals to it.  If the SIM has reported that it does not have any proactive capability then an STC: 0 unsolicited response will be sent to the application.

| Tab5 12 AT+STPD SIM Toolkit Command Response data |  |
| --- | --- |
| Set command +STPD=<length>,<data> | Response OK +CME ERROR: <err> +STC: 0 |
|  | Parameter <length > Integer Determines the number of bytes of <data> used for the Profile Download data from the application. <data> List Of Hex Values, two digits each: Hexadecimal representation of the Terminal Profile data (see [8]). |
| Reference | Note Some octets are optional in the profile, hence the inclusion of a length parameter. For example, the following command sets all the bits in octets 3 and 4: AT+STPD=4,0000FFFF. |

### AT+STEV SIM Toolkit Event Command

The application can inform the MS of defined MMI events using this command.

| Tab5 13 AT+STEV SIM Toolkit Event Command |  |
| --- | --- |
| Test command AT+STEV=? | Response +STEV: (supported <event> list) +CME ERROR: <err> |
| Set command +STEV=<event>,<language> | Response +CME ERROR: <err> |
|  | Parameter <event> hex two digits: 05 User Activity Event 06 Idle Screen Event 08 Language Selection Event 09 Browser Termination Event FF Clear Current Event List <language> string type up to two characters |
| Reference | Note The <language> parameter is applicable only to Language Selection Event. For example the language can be set by: AT+STEV=09,”11” |

### AT+STMS SIM Toolkit Main Menu Selection Command

The application may set up its main menu on receipt of the Set Up Menu SIM Toolkit event.  The application can select an item from the menu by sending this AT command to the MS.

| Tab5 14 AT+STMS SIM Toolkit Menu Selection Command |  |
| --- | --- |
| Test command AT+STMS=? | Response +STMS: (range of available <item>s),<0-1> +CME ERROR: <err> |
| Set command +STMS=<item>[,help] | Response +CME ERROR: <err> |
|  | Parameter <item> numeric type, giving unique identifier of menu item <help> numeric type |
| Reference | Note For example, AT+STMS=2,1 will select item 2 from the main menu with help. |

### AT+STRT SIM Toolkit Response Timer Command

When a proactive command is received from the SIM an automatic response timer is started. If this timer expires before the application has provided a suitable response via the +STCR command, a Terminal Response is sent to the SIM containing a result of No User Response. This AT command allows the automatic response timeout period to be configured by the application at run-time, thus giving it extended time to respond to certain proactive commands (e.g. the Get Input command may request a long input string to be entered as part of the associated test case).  The default setting for the response timer is ten seconds, and the maximum duration available is one hour.

| Tab5 14 AT+STRT SIM Toolkit Response Timer Command |  |
| --- | --- |
| Test command AT+STRT=? | Response +STRT: (list of supported <duration>s) +CME ERROR: <err> |
| Read command AT+STRT? | Response +STRT: <duration> +CME ERROR: <err> |
|  | Parameter See set command |
| Set command +STRT=<duration> | Response +CME ERROR: <err> |
|  | Parameter <duration> numeric type. Minimum = 1s, maximum = 3600s |
| Reference | Note Default setting is ten seconds |

### AT+STTONE SIM Toolkit Tone Command

The application may request a tone to played after receiving the Play Tone proactive command.  The application either starts playing the tone with the requested tone Id, or stops playing the current tone depending on the <mode> parameter. Tones may be played in either idle or dedicated mode.

On completion of the current tone, unsolicited result code +STTONE: 0 will be issued by the CI Task. However, if <mode>=0 is used to terminate the tone before it has completed playing there will be no unsolicited result code but only a result code of OK generated by the CI Task.

| Tab5 14 AT+STMS SIM Toolkit Menu Selection Command |  |
| --- | --- |
| Test command AT+STTONE=? | Response +STTONE: (list of supported <mode>s),(list of supported <tone>s),<list of supported <duration>s> +CME ERROR: <err> |
| Set command +STTONE=<mode>[,<tone>[,<duration>]] | Response +CME ERROR: <err> |
|  | Parameter <mode> 0 Stop playing tone 1 Start playing tone <tone> numeric type 1 Dial Tone 2 Called Subscriber Busy 3 Congestion 4 Radio Path Acknowledge 5 Radio Path Not Available / Call Dropped 6 Error / Special information 7 Call Waiting Tone 8 Ringing Tone 16 General Beep 17 Positive Acknowledgement Tone 18 Negative Acknowledgement or Error Tone <duration> numeric type, in milliseconds. Max requested value = 255*60*1000 = 15300000ms (supported range = 1- 15300000) |
| Reference | Note The default <tone>, if none entered, is General Beep. The default <duration>, if none entered, is 500ms. |